Eno Board – Basic Instructions

The Eno Interactive Whiteboard magnetic icon strip can be used to interact with anything showing on your computer screen, including a Word document, the Internet, or the image under the document camera.

- Turn on projector and computer inside presentation cart.

- Locate dark blue Eno Pen.

- Remove cap – placing it on the opposite end of the pen so it won’t get lost.

- Touch the tip of the pen to the Eno board. Try to use the pen like you would a mouse. Tap the Internet Explorer icon twice to open it. If the pen does not seem to respond you may need to recalibrate.

Projection Button

Back one page

Forward one page

Keyboard

Switch between draw and mouse

Select pen color

Pens of different colors

Select pen thickness from small to large

Different pen styles

Erasers – two sizes

Erase all

To bring up the Calibration screen click the Projection button. A yellow-green screen with an X in the top left-hand corner should appear. Press the tip of the pen in the center of the X. Do this for each X that appears.

Save

Print

- To add notes on top of what appears on your computer screen press the “Switch between” button. Then press a pen color and begin writing. (Note on our icon strip this button is at the top – or bottom depending on how the magnetic strip is placed on the board.) When you click the “Switch” button to go back to using the pen as a mouse the drawings will disappear. The drawings will reappear when you press the “Switch” button again.

- To erase some of what you’ve drawn on the screen tap one of the two sizes of erasers. Then drag the tip of the pen across the top of what you want to erase.

- To erase all of the drawing tap the “erase all” icon on the strip.

- To type in a Word document, open a blank Microsoft Word document. Tap the Keyboard button. Tap the Word document. Begin to type by tapping a letter.

- To save a copy of what you have drawn including the screen image tap the Save icon. Save your document as a .jpg file or .pdf.